Pressure Washing Business Starter Kit

File 14 — Pressure Washing Job Checklists | v1.0 | 2026-07-26

How to use this

Print these, laminate them, keep a set in the truck. A checklist isn't an insult to a good tech — it's how you deliver the same result whether it's you, your best guy, or someone on day three. Every callback you ever get will trace back to a line someone skipped.

Two rules run through all of it. Water goes top to bottom — always work down a surface so dirty runoff never crosses clean work. And the cleaning solution does the work, not the pressure — dwell time is the difference between a professional soft wash and a damaged wall. If you find yourself reaching for more pressure, you almost always need more dwell instead.

Every job runs the same three phases: pre-job walkthrough, the work, final walkthrough with the client. Skip the walkthroughs and you'll spend the money you saved on callbacks.

CREW STANDARDS — Every Job, Every Time

- Arrival: Arrive inside the window. Text the client on arrival and again at completion.

- Walkthrough first: Complete the Property Condition Acknowledgment (File 13) with the client. Photograph everything noted. If you didn't photograph it, you own it.

- Windows and doors: Confirm closed and secured before any water runs. Check the garage side door — it's the one everybody forgets.

- Protect first: Cover or pre-wet plantings, cover exterior outlets and fixtures, move or note anything in the work zone.

- Test spot: On any unfamiliar surface, test in an inconspicuous area and check the result before committing to the whole elevation.

- Right method, right surface: Soft wash where the material demands it. Pressure is a last resort on anything painted, aged, or wood.

- Runoff: Know where your water is going before you start. Containment and disposal rules vary by municipality — verify locally, especially for commercial flatwork.

- Final walkthrough: Walk the finished job with the client. Ask directly: "Anything you want me to look at again?"

- Photos: After photos on every job, every time. This is your marketing library and your dispute insurance in one.

- Leave it better: Coil hoses, pick up trash (yours and theirs), rinse overspray off walks and cars, close gates. The last impression is the one they describe to a neighbor.

PRE-JOB WALKTHROUGH — Every Job

☐  Confirm scope with the client out loud — what IS and ISN'T included today

☐  Complete Property Condition Acknowledgment; note and photograph pre-existing damage

☐  Identify water source, test flow and pressure, confirm you may use it

☐  Locate electrical panels, exterior outlets, low-voltage lighting, and any exposed wiring

☐  Confirm all windows, doors, and openings are closed and secured

☐  Confirm vehicles moved, furniture/grills/décor cleared or noted

☐  Note pets, children, and where they'll be during service

☐  Identify sensitive plantings and the runoff path

☐  Confirm parking and equipment staging that won't block the client or neighbors

☐  Set expectations out loud on anything that may not fully come clean

CHECKLIST 1 — HOUSE WASH (SOFT WASH)

The bread-and-butter job. Low pressure, correct solution, proper dwell. Pressure damages siding, forces water behind it, and strips paint — soft washing is the professional method for a house exterior.

Setup

☐  Confirm siding material and approximate age (vinyl, aluminum, wood, stucco, fiber cement, brick)

☐  Test spot in an inconspicuous location; confirm result before proceeding

☐  Mix solution to the strength appropriate for the surface and level of growth

☐  Pre-wet all landscaping thoroughly along the full work path

☐  Cover exterior outlets, light fixtures, and any exposed connections

☐  Confirm downspouts and grade will carry runoff away from the foundation

The wash

☐  Apply solution bottom to top — prevents streaking on the dry surface below

☐  Allow full dwell time; keep the surface wet, do not let solution dry on it

☐  Re-apply to heavy growth areas rather than increasing pressure

☐  Agitate stubborn areas by hand (soft brush) where needed

☐  Rinse top to bottom with low pressure, working in overlapping passes

☐  Rinse under eaves, soffits, and gutter faces

☐  Clean around and behind downspouts and utility penetrations

☐  Wash exterior window frames and sills; rinse glass so it dries clean

☐  Address door frames, porch ceilings, columns, and railings

☐  Rinse all landscaping again thoroughly at completion — this is not optional

☐  Rinse walks, drives, and any hardscape that caught overspray or runoff

Check before you call it done

☐  Walk every elevation and look up — missed spots hide under the eave line

☐  Check for streaking at downspouts and corners

☐  Confirm no solution dried on glass or trim

☐  Check neighboring property for overspray; rinse if needed

☐  After photos of each elevation

CHECKLIST 2 — CONCRETE & FLATWORK (DRIVEWAYS, WALKS, PATIOS)

Where a surface cleaner earns its price. The mistake that defines an amateur is visible wand stripes — inconsistent overlap that leaves zebra lines a client sees for years.

☐  Sweep or blow the surface clear of debris before any water

☐  Note and photograph existing cracks, spalling, oil stains, and prior discoloration

☐  Identify and pre-treat oil, rust, and organic staining with the appropriate product

☐  Confirm where runoff goes; contain or divert as required locally

☐  Pre-wet adjacent plantings and cover anything sensitive

☐  Apply cleaning solution and allow dwell — do not let it dry

☐  Clean with a surface cleaner at consistent speed and overlap; do not free-wand open areas

☐  Wand only the edges, corners, and transitions the surface cleaner can't reach

☐  Keep passes consistent — inconsistent speed shows as stripes once dry

☐  Rinse the full area, working with the slope

☐  Rinse residue off adjacent walks, curbs, and the street apron

☐  Post-treat remaining organic staining if needed

☐  Rinse landscaping again

☐  After photos taken after the surface has dried — concrete looks uniform when wet and tells the truth when dry

CHECKLIST 3 — ROOF (SOFT WASH ONLY)

Never, under any circumstance, put high pressure on a roof. It destroys shingles, voids manufacturer warranties, and turns a cleaning job into a roof replacement claim. If you aren't equipped and trained for roof work — including fall protection — decline the job.

☐  Assess roof age, material, and condition from the ground; photograph

☐  Decline if shingles are lifted, brittle, or visibly failing — document why

☐  Confirm fall protection and safe access; never work a wet roof without it

☐  Pre-wet ALL landscaping and the full runoff path generously

☐  Cover or divert downspouts as needed to control where solution lands

☐  Apply approved low-pressure solution evenly; allow dwell

☐  Do not scrub, brush, or blast — the growth releases and weathers off

☐  Re-apply to heavy areas rather than mechanically agitating

☐  Rinse or leave per the method used and the product's directions

☐  Rinse all landscaping thoroughly, twice

☐  Rinse siding, windows, walks, and drive of any runoff

☐  Clear gutters of released debris if included in scope

☐  After photos from the same angles as the before photos

CHECKLIST 4 — DECKS & FENCES (WOOD / COMPOSITE)

Wood is the easiest surface to permanently damage and the easiest to be blamed for. Low pressure, wide fan, consistent distance — always with the grain.

☐  Identify material: pressure-treated, cedar, redwood, hardwood, or composite

☐  Probe for soft or rotted boards; photograph and show the client before starting

☐  Note loose boards, popped nails, protruding screws, and railing wobble

☐  Test spot in a hidden area; confirm before proceeding

☐  Clear furniture, grills, planters, and rugs

☐  Pre-wet surrounding plantings

☐  Apply appropriate wood cleaner; allow dwell

☐  Clean at low pressure with a wide fan tip, consistent distance, always with the grain

☐  Never stop or dwell in one place — that's how you etch the wood

☐  Clean railings, spindles, stairs, and fascia

☐  Apply brightener/neutralizer if the process calls for it

☐  Rinse thoroughly; rinse landscaping

☐  Advise the client on dry time before furniture returns, and on sealing timeline if applicable

☐  After photos

CHECKLIST 5 — GUTTER FACE BRIGHTENING

Sold as an add-on and worth real money because "tiger striping" is invisible to the homeowner until you show them.

☐  Photograph the striping and show the client the before/after expectation

☐  Confirm scope: exterior face brightening ≠ gutter cleaning (interior debris removal)

☐  Apply gutter cleaning solution to the exterior face; allow dwell

☐  Agitate by hand with a soft brush — this job is hand work, not pressure

☐  Rinse top down, working the full run

☐  Verify no streaking remains at the seams and end caps

☐  Rinse siding and windows below

☐  After photos from the same angle — this is one of the best before/after sells you have

FINAL WALKTHROUGH — Every Job

☐  All equipment, hoses, and materials removed from the property

☐  Overspray rinsed from vehicles, walks, and neighboring surfaces

☐  Gates closed, pets accounted for, outlets uncovered

☐  Walk the job with the client; ask "anything you'd like me to look at again?"

☐  Address any concern on the spot if it's reasonable — a five-minute touch-up beats a callback

☐  After photos captured for every service performed

☐  Invoice presented or scheduled per your terms (File 10)

☐  Review requested while you're standing in front of the result (File 12)

☐  Next service discussed if the surface is on a maintenance cycle

☐  Completion report / photos sent before leaving the driveway